Photon Mono M5s - Firmware Update Guide

Version 1.0.4.0 Update Log:

- Added leveling inspection operation key

2. Optimized multiple details to enhance user experience"

1. Tools:

USB Drive

2. Update Steps:

Step 1: Copy Files to USB Drive

① Format the USB Drive with the "FAT32" file system and a "32KB" allocation unit size.

② Compress the firmware package.

③ Copy the file "ota-MonoM5s-1.0.4.5-A46246D7-20240606.acf" from the firmware folder to the USB Drive and safely eject the USB drive.

Step 2: Start the Device

Insert the power cable into the printer, turn on the power switch at the back of the device, and start the device.

Step 3: Insert USB Drive

After the device starts up to the main interface and the device's WIFI module is active, insert the USB Drive with the copied files into the USB port of the device.

Step 4: Update Firmware

On the device's main interface, click on the following menus: "Information" -> "More Settings" -> "Version" -> Upgrade icon -> "USB." The device will automatically start the firmware update (takes about 10 minutes).

Note:

① If the device stays on the "Firmware CRC Check!" screen during the update, try redownloading the firmware package and update from another USB Drive.

② After the update is complete, manually restart the device.

Step 5: Check Firmware Update

① Manually restart the device and wait for the WIFI module to fully start.

② On the device's main interface, follow these steps: click "Information" -> "More Settings" -> "Version."

③ Check if the serial number is "1035-2112-1012-2056-2006"

If the last set of numbers in the serial number is blank, the WIFI module has not fully started. Wait for a moment and check again.

If the serial number does not match the specified "1035-2112-1012-2056-2006" the update may have failed. Repeat the firmware update following the instructions to ensure a successful update.

If this firmware update guide is not helpful or if you encounter any issues during the update process, we recommend that you create a support ticket.

The after-sales engineers will assist you in finding a solution to the problem(Reply time :1 working day)

Please click this link to submit your ticket:

After-sales Ticket Submission Page